DS-K4H258S

One-Door Magnetic Lock

| The DS-K4H280S one-door magnetic lock is designed for wooden door, glass door, and steel door with the opening angle of 90°. The maximum thrust of the lock is 280kg. It can be used for controlling door opening/closing, and the indicator shows the door status. |  |
| --- | --- |

AVAILABLE MODEL

DS-K4H258S

MAIN FEATURES

- The magnetic lock supports static linear thrust of 280kg(550Lbs);

- Default voltage is 12VDC;

- It is equipped with internal voltage dependent resistor (MOV);

- It is applied to wooden door, glass door, metal door and fireproof door;

- LED indicator displays the status of door lock;

- Anti-residual magnetism design

- Abrasion-proof materials;

- The shell is made up of aldural and is hard anodizing electroplated;

- No mechanical failure, and the magnetic lock works by electromagnetism force;

SPECIFICATIONS

| Model | DS-K4H258S |
| --- | --- |
| Dimension of Lock Body (L×W×H) | 238×45×25mm (9.4×1.8×1.0") |
| Dimension of Armature Plate (L×W×H) | 182×38×12.5mm (7.2×1.5×0.5") |
| Unlock Method | Current Interruption |
| Maximum Thrust | 280kg(550Lbs) Linear Thrust |
| Input Voltage | 12 VDC |
| Working Current | 340mAx2 |
| LED Indicator | Green (Work Status) |
| Suitable Door | Wooden Door, Glass Door, Metal Door, Fireproof Door |
| Surface Temperature | ≤ Ambient Temperature +20°C (68°F) |
| Working Temperature | -10°C~+55°C (14°F~131°F) |
| Working Humidity | 0~95% (Relative Humidity) |
| Shell | Hard Anodizing Electroplating Operated |
| Lock Body | Eco-friendly Nickel Electroplating Operated |
| Armature Plate | Eco-friendly Zinc Electroplating Operated |
| Weight | 2.1kg |